Name:___________________________________

Date:____________________________________

Unit:____________________________________

For the following questions, circle the choice that best answers the question.

- ECV HMMWVs and FPI CAT Is & IIs can operate up to _____ degrees side slope?

  - 13

  - 19

  - 25

  - 31

- What obstacle may make egress from a side door very difficult or impossible after a rollover when the vehicle comes to a rest on its side?

  - Combat door locks are likely to jam.

  - The time consuming need to doff gear.

  - Damage to the turret assembly blocking a door.

  - The weight of the doors when the vehicle is in this position.

- What should you do when establishing your egress plan?

- Get a paper and pencil.

- Identify the closest roof and side exits, when applicable.

- Avoid exits that are small or have combat door locks.

- Anticipate how the vehicle will come to a rest after a rollover.

- Which of the following should you do before you start driving to help avoid rolling your wheeled vehicle?

  - Increase tire pressure above specification, keep CG high, and ensure payload is under vehicle specification.

  - Decrease tire pressure below specification, keep CG high, and ensure payload is over vehicle specification.

  - Ensure tire pressure is within specification, keep CG low, and ensure payload is under vehicle specification.

  - Ensure tire pressure is within specification, keep CG low, and ensure payload is over vehicle specification.

- Which of the following is one thing drivers should do to help avoid a rollover when cresting a hill?

  - Avoid going over hills.

  - Increase speed going over hills.

  - Decrease speed going over hills.

  - Keep a constant speed over hills.

- Which of the following should drivers do to help avoid a rollover when the vehicle starts to run off the edge of the road?

  - Speed up and quickly turn steering wheel back to the road.

  - Speed up and slowly turn steering wheel back to the road.

  - Gently slow down and quickly turn steering wheel back to the road.

  - Gently slow down and slowly turn steering wheel back to the road.

- Where do the majority of rollovers occur?

- On rural roads

- On highways

- In city areas

- On bridges

- Which of the following is not something you can do to help avoid a rollover?

- Activate rollover prevention system.

- Avoid driving too close to the edge of roads.

- Incorporate potential for rollovers in risk assessment.

- Proper PMCS and pre-operations checks prior to departing on a mission.

- What do you yell when a rollover is imminent?

  - Water, Water, Water!

  - Danger, Danger, Danger!

  - Rollover, Rollover, Rollover!

  - We’re all going to die!

- What should the driver do when a rollover is imminent?

  - Unlock combat doors.

  - Continue to drive and navigate the vehicle.

  - Vigorously turn steering wheel to avoid rolling over.

  - Let go of the steering wheel and tuck chin into chest.

- What should the Gunner do during a rollover?

  - Fasten seatbelt.

  - Grip the weapon firmly.

  - Jump away from the vehicle.

  - Push or pull self down into vehicle.

- Which of the following is one thing that all crewmembers should do during a rollover?

  - Unlock combat doors.

  - Grip their weapon firmly.

  - Push the Gunner away from the vehicle.

  - Assume a proper brace position.

- What should the back seat crew, closest to the Gunner, do to help the Gunner during a rollover?

  - Fasten the Gunner’s seatbelt.

  - Pull the Gunner inside the vehicle.

  - Push the Gunner away from the vehicle.

  - Move out of the way to give the Gunner room.

- After a dry rollover, what should you do immediately before you unbuckle your seatbelt?

  - Assess injuries.

  - Take your helmet off.

  - Orient yourself to a door.

  - Brace yourself and protect your neck.

- What does the first Marine to egress after a dry rollover do?

  - Cover the exit while the other Marines egress.

  - Help the second Marine egress injured Marines.

  - Post security at the front of the vehicle.

  - Call for support.

- After a 180 degrees dry rollover, where should the 2nd Marine to egress post security?

  - 3 o’clock position relative to the egress point

  - 6 o’clock position relative to the egress point

  - 9 o’clock position relative to the egress point

  - 12 o’clock position relative to the egress point

- Which of the following are some of the most common symptoms of heat exhaustion?

  - Fast and shallow breathing, muscle cramps, dizziness

  - Flushed skin, hives, loss of vision

  - Hot sweats, hallucinations, dizziness

  - Dry skin, dry mouth, dilated pupils

For the following questions, circle all of the choices that answer the question.

- Which of the following are components of a good brace position?

- Seat-belted body

- Head tucked into chest

- Knees tucked into chest

- Arms tucked in close to body, firmly gripping something stationary

- How do you regain your orientation after the violent motion of a rollover stops?

- Keep seatbelt engaged and hold onto reference

- Sit up straight

- Disengage seatbelt

- Look/recall where you are relative to the rest of the vehicle

- After a dry rollover, what should you do before releasing your seatbelt to reduce the chances of injuring one of your fellow crewmembers?

- Determine if you can open a door from your seat.

- Determine if another Marine is/may be located in your landing spot.

- Attempt to communicate with Marine in your landing spot.

- Allow them to get out of their seatbelt first in order to help you get down.